Second Midterm		     CS3744		20 April, 2010

Name:____________________________________

(30) 1. Give brief definitions (not examples) for the following terms in the spaces provided:

a) Homogeneous coordinates

b) Convex polygon

c) Affine linear combination

d) Windowing transformation

e) Aliasing

f) Vector

(10)  2.  Suppose we have 105 points and we wish to determine as quickly as possible which

are inside and which are outside the triangle below.  Show in specific detail how to

do the computations for this triangle.  To demonstrate, determine whether the point

(6.2, 3.9) is inside or outside.

Hint:   A 2-point form for the equation of a line is

(y2 – y1) x + (x1 – x2) y + (x2y1 – y2x1) = 0.

(10)  3.  For the figure below, determine the matrix form of the instance transformation:

10) 4. The Bresenham algorithm involves the decision variable, d, where d0 = y - x/2 and

dk+1	= dk + y -  x	dk > 0

= dk + y 		otherwise

If dk > 0 then NORTHEAST else EAST at step k.

Calculate the first 6 chain codes for the line from (-10, 0) to (20, 60).  Show your

computations at each stage.  Specify your chain code directions.

(10)  5. Shade the interior regions of the following polygon using the winding rule:

(10)  6.  The figure below shows a finite line segment with different intensities

at the endpoints.

a) Determine the (a, b, c) vector for this line.

b) Determine the intensity of the marked point on the line by linear interpolation.

(5)  7.   Suppose a line is given by the equation (1, -6, -2)  (x, y, 1) = 0.  Write the equation

of this line in slope-intercept form.

(10)  8.  Suppose we are given the line (a, b, c)  (x, y, 1) = 0.

a)	What is the slope of this line?

b)	What is the y-intercept of this line?

c)	Determine a normal vector for this line.

d)	Determine a perpendicular to the normal vector for this line.

e)	Explain the difference between a vector and a line

(5)  9.  Determine the cosine of the angle between the vectors  (1, 4, 2) and (0, 0, 1).